C1 | Compressor shuts down intermittently after ~2 minutes of run time; thermal overload relay trips. | Presenting Symptom / Diagnostic Trigger | The main symptom; indicates a conflict between motor demand and thermal protection limits.

C2 | Motor housing is measured at 92 °C via IR gun; unusually hot to the touch. | Diagnostic / Symptom | Confirms a genuine thermal event is occurring, ruling out a simple electrical "nuisance" trip.

C4 | At the moment of thermal shutdown, the manual gauge shows tank pressure is 20–25 psi below the 175 psi cut-out threshold. | Diagnostic | Shows the compressor is running but failing to reach the target shut-off pressure before overheating.

C5 | Compressor startup sounds heavy and strained, as if starting against high internal pressure. | Diagnostic / Auditory | Indicates the motor is starting "under load" rather than being relieved by the unloader valve.

C7 | Ambient room temperature is stable at 25 °C and the motor cooling fan is functional. | Neutral | Eliminates environmental factors or fan failure as the cause of the 92 °C motor temperature.

C8 | Replacing the thermal overload relay (LRD 32) with a new unit did not resolve the shutdown pattern. | Confirmatory / Maintenance | Rules out a faulty relay; confirms the motor is drawing actual overcurrent or generating real excess heat.

C9 | Compressor oil level is full, oil is clean, and there is no measurable vibration or shaft wobble. | Neutral | Eliminates mechanical friction, lack of lubrication, or physical misalignment as heat sources.

C10 | Air intake filters and vent ports are clean; no obstructions found in the suction lines. | Neutral | Confirms the compressor is not struggling due to intake restrictions or "vacuum loading."

C11 | A new 15 SCFM cylinder press was recently added to the system without updating control logic. | Contextual / Distractor | Explains the increased total air demand but is not the root cause of the motor overheating.

C13 | Digital pressure switch is programmed for a 125 psi cut-in and 175 psi cut-out. | Neutral / Logic | Defines the expected electronic thresholds for the 24VDC control signal.

C15 | 3-phase power delivery to the motor is stable; wiring in the control cabinet shows no loose or oxidized contacts. | Neutral | Rules out electrical supply issues, such as single-phasing, as the cause of the heat build-up.

C16 | Floor operators report that the frequency of compressor start-up cycles has increased recently. | Operational | Confirms the system is cycling more often, which contributes to the thermal load on the motor.

C18 | Resetting the thermal relay allows the system to restart immediately once cooled, but the cycle repeats. | Diagnostic | Confirms the fault is repeatable and tied to the run-cycle duration, not a permanent component failure.

C19 | The digital pressure sensor outputs a steady 24VDC signal when the display reads below 125 psi. | Neutral | Confirms the electronic logic is functioning as designed based on the (incorrect) sensor data.