"cues": [

{

"id": "S1",

"cue": "LED never lights when button is pressed",

"type": "PresentingSymptom",

"associated_measurement": "Visual observation: no light under any condition",

"interpretation": "Indicates a fault exists but does not localize the path",

"category": "Symptom",

"socratic_prompts": [

"Where in the chain does voltage stop?"

],

"divergent_thinking_prompts": [

"List 3 places an open could occur besides the LED itself."

],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "S2",

"cue": "LED briefly flickers when wires are wiggled",

"type": "PresentingSymptom",

"associated_measurement": "Visual observation: intermittent light when mechanically disturbed",

"interpretation": "Suggests intermittent/open connection on breadboard row",

"category": "Symptom",

"socratic_prompts": [

"What single measurement would turn this suspicion into evidence?"

],

"divergent_thinking_prompts": [

"Imagine three micro-failure modes that cause intermittency (crack, bent clip, oxidation). How would you differentiate them?"

],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "P1",

"cue": "Board power rail voltage is nominal",

"type": "Neutral",

"associated_measurement": "Multimeter: +3.3 V (or +5 V) at Vcc rail; 0 V at GND rail",

"interpretation": "Confirms overall power is present",

"category": "Power",

"socratic_prompts": [

"Given power is good, which subpath should you test next?"

],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "P2",

"cue": "MCU 3V3/5V pin reads nominal",

"type": "Neutral",

"associated_measurement": "Multimeter: expected rail voltage at MCU power pin",

"interpretation": "MCU supply OK; look downstream",

"category": "Power",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "P3",

"cue": "Total supply current is very low when LED should be on",

"type": "Diagnostic",

"associated_measurement": "Series ammeter: near 0\u20135 mA instead of expected LED current",

"interpretation": "Open circuit in LED path likely (e.g., broken strip)",

"category": "Power",

"socratic_prompts": [

"If current is near zero, which segment likely contains an open?"

],

"divergent_thinking_prompts": [],

"evidence_required": [

"Measure current with series ammeter; compare to expected I_LED"

],

"confirmatory_actions": []

},

{

"id": "L1",

"cue": "LED anode node reads 0 V when LED should be ON",

"type": "Diagnostic",

"associated_measurement": "Multimeter: 0 V at LED anode; upstream node commanded HIGH",

"interpretation": "Open between upstream high and LED anode (broken strip likely)",

"category": "SignalPath",

"socratic_prompts": [

"Where exactly does the voltage drop disappear along the row?"

],

"divergent_thinking_prompts": [],

"evidence_required": [

"Probe upstream hole-by-hole along the same row"

],

"confirmatory_actions": []

},

{

"id": "L2",

"cue": "LED cathode node is ~0 V",

"type": "Neutral",

"associated_measurement": "Multimeter: ~0 V at LED cathode",

"interpretation": "Ground side is OK; fault is likely on anode side",

"category": "SignalPath",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "L3",

"cue": "Series resistor value is correct",

"type": "Neutral",

"associated_measurement": "Ohmmeter: ~330 \u03a9 (\u00b15%) or course-appropriate value",

"interpretation": "Rules out over-resistance as cause",

"category": "Component",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "L4",

"cue": "LED orientation is correct",

"type": "Neutral",

"associated_measurement": "Diode test: LED forward drop nominal (e.g., 1.8\u20132.2 V red)",

"interpretation": "Polarity OK; look for open in path",

"category": "Component",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "L5",

"cue": "LED works in a separate test circuit",

"type": "Neutral",

"associated_measurement": "Bench supply with series resistor lights LED",

"interpretation": "Component is functional",

"category": "Component",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "B1",

"cue": "Button electrical state toggles correctly",

"type": "Misleading",

"associated_measurement": "Logic probe/multimeter at input node: LOW\u2194HIGH on press",

"interpretation": "Tempts a software or input-focus; path to LED may still be open",

"category": "Input",

"socratic_prompts": [

"Does a correct input guarantee load current? Why or why not?"

],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "C1",

"cue": "Continuity across suspect breadboard row fails",

"type": "Diagnostic",

"associated_measurement": "Ohmmeter/continuity: open line / no beep between holes believed common",

"interpretation": "Direct evidence of broken strip",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [

"Remove power; measure ohms across row positions"

],

"confirmatory_actions": [

"Bridge the row with a short jumper and retest"

]

},

{

"id": "C2",

"cue": "High resistance (>1 M\u03a9) between two holes on same row",

"type": "Diagnostic",

"associated_measurement": "Ohmmeter: >> 1 M\u03a9 instead of <1 \u03a9",

"interpretation": "Indicates discontinuity in spring contact",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "C3",

"cue": "Voltage present at one hole of a row but absent at another hole of same row",

"type": "Diagnostic",

"associated_measurement": "Multimeter: e.g., 3.3 V at hole A; 0 V at hole B (same row)",

"interpretation": "Classic signature of a split/broken strip",

"category": "Continuity",

"socratic_prompts": [

"Map the row: where does the potential step occur?"

],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "C4",

"cue": "Mechanical feel: jumper wiggles loosely in specific row",

"type": "Diagnostic",

"associated_measurement": "Tactile observation: poor retention in the hole",

"interpretation": "Suggests damaged/bent clip or cracked strip",

"category": "Mechanical",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "C5",

"cue": "Back adhesive/metal bus shows crack or corrosion",

"type": "Diagnostic",

"associated_measurement": "Rear inspection: visible crack/oxidation under board",

"interpretation": "Physical root-cause evidence",

"category": "Mechanical",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "C6",

"cue": "Adjacent rows behave normally",

"type": "Neutral",

"associated_measurement": "Ohmmeter: adjacent rows show <1 \u03a9",

"interpretation": "Localizes defect to specific row only",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "T1",

"cue": "MCU output pin is HIGH when LED should be ON",

"type": "Neutral",

"associated_measurement": "Multimeter at header pin: expected HIGH level",

"interpretation": "Source drive exists; break is downstream",

"category": "SignalPath",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "T2",

"cue": "Voltage at wire end inserted into breadboard (same row as MCU pin) is HIGH",

"type": "Neutral",

"associated_measurement": "Multimeter: e.g., 3.3 V at that hole",

"interpretation": "Upstream wiring and insertion OK at entry point",

"category": "SignalPath",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "T3",

"cue": "Voltage one hole away (same row) is LOW",

"type": "Diagnostic",

"associated_measurement": "Multimeter: 0 V one hole away on same row",

"interpretation": "Break within the row strip between those points",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "T4",

"cue": "Resistor input lead (connected to suspect row) is LOW",

"type": "Diagnostic",

"associated_measurement": "Multimeter: 0 V at resistor lead",

"interpretation": "Break lies between upstream HIGH and resistor lead",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "T5",

"cue": "Resistor output lead is LOW",

"type": "Neutral",

"associated_measurement": "Multimeter: ~0 V",

"interpretation": "Downstream consequence of upstream open; not localizing by itself",

"category": "SignalPath",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "G1",

"cue": "Ground continuity from LED cathode to GND rail is good",

"type": "Neutral",

"associated_measurement": "Ohmmeter: <1 \u03a9 LED cathode \u2192 GND",

"interpretation": "Rules out ground open",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "G2",

"cue": "GND rail to MCU GND continuity is good",

"type": "Neutral",

"associated_measurement": "Ohmmeter: <1 \u03a9",

"interpretation": "Rules out ground architecture issues",

"category": "Continuity",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "M1",

"cue": "Breadboard section shows wear from repeated insertions",

"type": "Neutral",

"associated_measurement": "Visual: widened holes, bent contacts",

"interpretation": "Explains likelihood of contact failure",

"category": "Mechanical",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "M2",

"cue": "Pressing down on components causes momentary operation",

"type": "Diagnostic",

"associated_measurement": "Observation: LED flashes when pressing down",

"interpretation": "Contact temporarily re-makes across crack",

"category": "Mechanical",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": [

"Add mechanical pressure or shim; does it restore function?"

]

},

{

"id": "X1",

"cue": "Changing the insertion row resolves the issue",

"type": "Confirmatory",

"associated_measurement": "Action: Move LED/resistor to a new row \u2192 LED works",

"interpretation": "Confirms original row fault",

"category": "Confirmation",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "X2",

"cue": "Bridging the row with a short jumper restores operation",

"type": "Confirmatory",

"associated_measurement": "Action: Add short wire across suspected break \u2192 LED lights",

"interpretation": "Functional confirmation of discontinuity",

"category": "Confirmation",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "X3",

"cue": "Replacing the breadboard resolves the issue",

"type": "Confirmatory",

"associated_measurement": "Action: Transplant circuit to another board \u2192 works",

"interpretation": "Strong confirmation of breadboard defect",

"category": "Confirmation",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "X4",

"cue": "Voltage drop across LED+resistor nominal after fix",

"type": "Confirmatory",

"associated_measurement": "Multimeter: V_LED \u2248 spec; V_R consistent with expected current",

"interpretation": "Indicates normal current now flows",

"category": "Confirmation",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "D1",

"cue": "PWM ripple or button bounce visible on scope at input",

"type": "Misleading",

"associated_measurement": "Oscilloscope: small transients present",

"interpretation": "Not causal for a steady HIGH; LED would still light if path were intact",

"category": "Input",

"socratic_prompts": [

"How would you prove that ripple is not the primary cause?"

],

"divergent_thinking_prompts": [

"Design a test to average the signal (RC) and see if outcome changes."

],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "D2",

"cue": "Ambient light is bright, LED appears dim",

"type": "Misleading",

"associated_measurement": "Observation: hard to see LED in sunlight",

"interpretation": "Visibility artifact; does not prevent LED conduction",

"category": "Environment",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "D3",

"cue": "Trying different LED colors still off",

"type": "Misleading",

"associated_measurement": "Observation: red/green/blue all off",

"interpretation": "Open path dominates; LED choice irrelevant",

"category": "Component",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "D4",

"cue": "Swapping to a lower-value resistor has no effect",

"type": "Misleading",

"associated_measurement": "Ohmmeter confirms new value; LED still off",

"interpretation": "Open circuit dominates; resistance value irrelevant",

"category": "Component",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

},

{

"id": "D5",

"cue": "Power rail indicator LED on regulator module is lit",

"type": "Misleading",

"associated_measurement": "Observation: rail LED is on",

"interpretation": "Only proves rail power; not path continuity to your LED",

"category": "Power",

"socratic_prompts": [],

"divergent_thinking_prompts": [],

"evidence_required": [],

"confirmatory_actions": []

}